1. Copy out the table opposite and then

classify each of the terms in the left-hand

column by placing a tick in the relevant

box. [5]

2. Complete each of the following statements in words:

The resistance of an ammeter is assumed to be …….

The resistance of a voltmeter is assumed to be …….. [2]

Calculate the total resistance of four 5.0 Ω resistors connected in parallel. [2]

3. A negative temperature coefficient thermistor is used in the

circuit opposite to make a temperature sensor.

Explain how the circuit works. [2]

The graph shows how the resistance of the thermistor

varies with temperature.

What will the reading on the milliammeter be when

the thermistor is at a temperature of 20°C? [3]

4. A copper wire has a cross-sectional area of

0.20 x l0-6 m2.  Copper has 1.0 x 1029 free

electrons per cubic metre.

Calculate the current through the wire when

the drift speed of the electrons

is 0.94 mm s-1. [3]

The wire is 4.0m long. Copper has a resistivity

of 1.7 x l0-8 Ωm. Calculate the resistance of

the wire. [3]

Calculate the potential difference across the

wire. [1]

A second wire with the same dimensions is

made from a material that has a greater

resistivity than copper. Explain how, if at all, the

current will differ from that in the copper wire

when the same p.d.is applied across it. [2]

The number of free electrons per cubic metre in

this wire is the same as that in the copper wire.

Compare the drift velocities of the free electrons in

the two wires. [1]

5. A heater, for use outdoors, uses bottled gas to

make a metal grill red hot. The metal grill is

mounted beneath a polished aluminium hood.

Briefly explain the advantage of having an

aluminium hood which is polished

(i) on the underside

(ii) on its upper surface. [4]

QUESTION 5 CONTINUES ON THE NEXT PAGE

Q5 continued….

The bottled gas provides energy for approximately 16 hours when working at 14.4 kW power. Show that the total energy provided during this time is about 800 MJ. [3]

This device is 45% efficient at heating the seating area. Calculate the wasted energy. [3]

Explain why the efficiency of the heater will be less than 45% when it is first switched on [2]

6.  A student wants to determine the e.m.f. E of a cell and its

internal resistance r.

He uses the circuit shown and measures the terminal

voltage V across the cell and the current I in the circuit for

each setting of the variable resistor.

He plots the graph shown of terminal voltage V against

current I.

Show how the relationship V = E - Ir

can be used with his graph to determine the e.m.f. E of

the cell. State its value. [2]

Show how the graph can be used to determine the internal

resistance r of the cell. Calculate its value. [2]

The student repeats the experiment using

two of these cells in series. On  a copy of this

graph, draw the line that he obtains. [3]

Suggest why the student includes the

filament lamp in the circuit. [2]

7. If you want to investigate how the

pressure of a gas depends on the volume

of the gas, two variables must be kept

constant. What are they? [2]

Draw a labelled diagram of the apparatus

you would use. [3]

How would you process the readings you have taken in order to

produce the graph shown above on the left?

Copy this graph and label both axes. [2]

8. The contents of a domestic refrigerator are at a constant temperature of 5 °C,

and the outside surface of the refrigerator is at a constant temperature of 20 °C.

Explain how it is possible for the contents of the refrigerator to be at a constant

temperature even though energy is continuously flowing in from outside.

You may be awarded a mark for the clarity of your answer. [3]

The equation ΔU = ΔQ + ΔW can be applied to the contents of the refrigerator.

What is the value of ΔU ? Explain your answer. [2]

What is meant by ΔQ in this equation? [2]

Explain why ΔW is zero. [1]