132		Identification of device(s) from

their I/V characteristics

Apparatus:

Puzzle box labelled XY, Lab power supply unit, 2 x digital meters, Rheostat, Wires.

Procedure:

- Set the power supply to 6V.

- The puzzle box labelled XY contains a diode, a 10 resistor and another resistor connected together in one of the following ways:

X

Y

R2

R1

X

Y

R2

R1

X

Y

R2

R1

X

Y

R2

R1

A

B

C

D

- Set up a circuit to measure the current flowing from X to Y for potential differences between 0V and 5V.

- Reverse the power supply connections and obtain current values for potential differences down to -5V.

5.	Draw a current against voltage graph for the puzzle box for potential differences between –5V and +5V.

6.	Draw the circuit diagram of the set up you have used.

7.   	Use your graph to identify / calculate:

    - which combination of A, B, C or D is correct.

    - the values of resistances R1 & R2 (remember that one of them is 10).

- Explain how you obtained your answers to part 7.

9.	Add to your graph the curves that you would have expected to have obtained from the other three possible combinations.