# PHY 2 – Electricity revision –MAL

| ## Quantity | Unit | Definition | Meter |
| --- | --- | --- | --- |
| Current ( I) | Amp (A) | Charge/time | Ammeter in series |
| P.D. ( V) | Volt (V) | Energy delivered/charge | Voltmeter in parallel |
| Resistance (R) | Ohm ( ) | p.d./current | Ohm meter |

| Circuit | Current | P.D. | Resistance |
| --- | --- | --- | --- |
| Series | Same |  resistance | Increases R = R1 + R 2 |
| Parallel |  1/ resistance | Same | 1/R = 1/R1 + 1/R2 |

| Material | Drift velocity | Temperature coefficient of resistance |
| --- | --- | --- |
| Metals | Few mm s-1 | Positive |
| Semi conductors | Small | Negative |
| Insulators | Low | Needs v high pd to break down |

Current (I) = Number of charge carriers per unit volume(n) x c/s area(A) X drift velocity (v)  x electron charge( e)

## Emf ( E) – Value of p.d. when NO current delivered to circuit

Internal resistance ( r) – due to internal components of psu ( low for cell – 1 , high for High tension supply).

Electrical energy (E) = current (I) x p.d.(V) x time ( t)

Energy dissipated as heat = I 2 x R x t

Different devices have a different I – V characteristic as their resistance may vary with conditions –

Metal filament bulb – R increases with TEMPERATURE ( Curve)

Metal wire – R constant if temperature constant ( line)

Thermistor – R decreases with temperature ( curve)

LDR – R decreases with intensity of light ( curve)

Diode – R high at first then decreases