# E6

## Resistivity Calculations

- A copper track on a printed circuit board is 2.0 mm thick, 8.5 mm wide and 4 cm long. The resistivity of copper is 1.7 x 10-8 m. Calculate the track’s resistance. Find the voltage drop across it when it carries a current of 25mA.

- An electric fire has a power output of 1.2 kW and is designed to operate on the 240 V mains. Calculate (a) the resistance of the element, and (b) the length of wire needed to make the element, given that its resistivity is 3.3 x 10-4 m and it has a cross-sectional area of 3.0mm2.

- The electrical resistivity of manganin is 45 x 10 –8 m. Calculate the resistance of 2m of this wire which has a diameter of 1mm .

- Calculate the length of wire of 1.0 mm diameter and 5.0 x 10-6 m resistivity that would have a resistance of 5.0.

- An electrical heating element is to be designed so that the power dissipated will be 750 W when connected to the 240 V mains supply.

  - Calculate the resistance of the wire needed.

  - The element is to be made from nichrome ribbon 1.0 mm wide and 0.050 mm thick. The resistivity of nichrome is 1.1 x 10 –6 m. Calculate the length required.

  - Draw a circuit diagram to show how a second heating element would be connected to increase the power dissipated to 1.5kW. Explain your choice.

  - State one important property of a conductor used to make heating elements.