Data logger set up instructions for 025D

1.	Connect the DL PLUS 128 to the serial port of the PC. (see diagram)

A Chemistry wire may need to be removed.

2.	Attach a power supply to the DL PLUS 128.

3.	Attach another power supply to the DISTANCE sensor with its ultrasonic 				transmitted attached. Set the pulse rate on this sensor to 20s-1. It should emit clicks 	when switched on.

(the junction box is not needed for this experiment).

4.	Using a DIN lead to connect the distance sensor to channel 1 of the DL PLUS

5.	Turn on the PC.

Other apparatus needed for 025D:

Clamp stand, boss and clamp, G-clamp, spring with 500g mass/card attached, metre ruler.

If the computer is not available then supply experiment 002.