141	Measurement of capacitance

using a coulombmeter

Apparatus:

Two mounted capacitors marked A & B, coulombmeter, digital voltmeter set on 20V DC,

lab power supply set on 10V DC, rheostat, wires.

Circuit diagram:

V

C

10V DC

power

supply

coulombmeter

capacitor

rheostat

used as a

potential

divider

connection P

connection S

Procedure :

1.	Set up the circuit shown above with one of the capacitors.

The wire from the top of the capacitor should be initially connected to "connection P".

2.	Adjust the potential divider to give a voltage of about 5V. The capacitor will now be charged to this 	voltage. Record the actual voltage reading, V.

3.	Press the red button on the coulombmeter so that it reads zero nanocoulombs.

4.	Transfer the wire from "connection P" to "connection S". The charge that was on the capacitor will 	now be transferred to, and measured by, the coulombmeter. Record the charge reading, Q.

5.	Repeat stages 2 to 4 for a range of voltages between 0 and 10V.

6.	Repeat stages 2 to 5 for the other capacitor.

7.	Using a common set of axes plot graphs of charge Q against potential difference V for both capacitors.

8.	Use these graphs to answer the following:

(a) What do the graphs show about how the charge of a capacitor relates to its potential difference?

(b) Calculate the capacitance of each capacitor. (Remember that C = Q / V)

[Note: Capacitor A should be about 22nF; B about 47nF]

9.	When two capacitors are connected together in parallel the total capacitance CT is given by:

CT = C1 + C2

Use your circuit to check how well this relationship works with your capacitors.