145	Parallel Plate Capacitor

Apparatus:

EHT power supply, coulombmeter, two metal plates on insulating handles, 50cm ruler, two clamp stands with bosses and clamps, crocodile clip, long wires.

Procedure:

1.	Assemble the apparatus shown above.

Ensure: 	(a) that the positive flying lead is connected to the lower positive 					terminal (the one that protects YOU with a 50M¦ resistor).

(b) that the negative terminal of the power supply is connected to the 				Earth terminal.

2.	Place the capacitor plates facing each other so that they are  d = 1 cm  apart.

3.	Adjust the output of the EHT (Extra High Tension) supply so that its voltmeter reads 	2000V.

4.	Charge the capacitor by momentarily touching the unearthed (right-hand) plate with 		the flying lead.

5.	Measure the charge stored in the following way:

(a) Reset the coulomb-meter by pressing the red button, the meter should

now read 0 nanocoulombs (nC). [ 1nC = 1.0 x 10 - 9 C]

(b) Move the meter towards the right-hand plate, touch the plate with the collecting 		(metal) cap. The meter should now read the charge Q stored by the capacitor.

6.	Repeat stages 2 to 5 for five further plate separations d between 0.5 cm and 6.0 cm.

7.	Calculate 1	and tabulate this along with d and Q.

d

8.	Plot a graph of Q against  1 .

d

9.	Measure the gradient G of your graph.

10.	Measure the area A of one of the plates.

11.	The capacitance C of a parallel plate capacitor is given by:

C = 	Q		& 	C =   A

V				  d

where:  = the permittivity of the space between the plates.

12.	Show that the two equations above can be used to obtain the expression:

Q  =  AV

d

13.	Use the above expression and the value of your graph’s gradient G

to calculate the permittivity,  .

[Hint: Y = MX  is the equation of a straight line graph through the origin.]