Resistance wire and variable resistors

1.	Obtain the following apparatus: [EC3]

2 x cells in holders, 1 x bulb in holder, 7 x wires, 6 x crocodile clips, multimeter, metre ruler, 	resistance wire, and a rheostat.

2.	Wire up the circuit shown below with about 10 cm of resistance wire between the crocodile clips.

multimeter set at 10A

10A

10

A

DC AC

COM

Multimeter

set at 10A

resistance wire

+

+

light bulb

A

showing terminals used

crocodile clips

3.	Notice what happens to the bulb's brightness and the ammeter reading when you vary the length of the resistance wire between the crocodile clips. The wire is called resistance wire because it resists the flow of electric current through it. The longer the wire, the greater is its resistance and the lower the current and brightness of the bulb.

4.	Now wrap the resistance wire around a pencil as shown in the diagram below.

Make sure that the turns of the coil do not touch each other.

Connect one crocodile clip to one end of the wire and touch (DO NOT CLIP) the wire at different 	points using the other crocodile clip. Notice again what happens to the bulb and the ammeter.

to bulb

to ammeter

resistance wire wound around a pencil

touching crocodile clip

5.  	The resistance wire and pencil make up a simple variable resistor.

The longer the wire used; the higher is the resistance and the dimmer is the bulb because the lower is 	the current flowing.

6.    	Now replace the resistance wire and crocodile clips in your circuit with the rheostat as shown in the

diagram on the next page. The rheostat is a more robust version of your pencil variable resistor.

+

+

A

+

+

A

slider

terminal A             terminal B

Slider terminal

7.  	In your note book write the heading "Using a variable resistor” Draw the circuit diagram (right-		hand diagram) in your note book.

8.  	Move the sliding contact of the rheostat and notice what happens to the bulb and ammeter.

Can you see the resistance wire of the rheostat? Which part of the rheostat is equivalent to the fixed crocodile clip connection with your pencil variable resistor? Which part of the rheostat is equivalent to the moving crocodile clip connection with your pencil variable resistor? What effect does moving the wire going into terminal A to terminal B?

9.	Copy the following into your note books, inserting the missing words.

Missing words:  lower; resistance; rheostat; current

A variable resistor is a device that can be used to control the level of ****** flowing in a circuit. In a

simple series circuit, the higher the ******* setting the ******* is the current.

A ******* is a device made of resistance wire that can be used as a variable resistor.

DRAW THE SYMBOL

200

V

Ω

COM

The symbol of a variable resistor is:

10.	Wire up the circuit shown opposite. The ohmmeter

will measure the resistance of the resistance wire in

between the two crocodile clips. Measure this resistance

for about eight different lengths of wire between 20cm

and 90cm. Present your results in a table in your note

book with the headings: length of wire (cm) & resistance (Ω)

11.	Draw a graph of resistance (Ω) [Y-AXIS] against length of wire (cm) [X-AXIS]. Use your 		points to draw a best-fit straight line through the origin.

Measure the gradient (slope) of your line (= Y-step / X-step).

12.	The gradient is equal to the resistance of the wire per centimetre. Attach the graph to your note book 	and complete the following sentences:

The resistance of the resistance wire is equal to __________ohms per centimetre.

The resistance is proportional to the length. This is shown by the graph being a _______ 		line that passes through the ______.