ELECTRICAL POWER, COST & DISTRIBUTION

11.3 Why are electrical devices so useful?

Using skills, knowledge and understanding of how science works:

•  to compare and contrast the particular advantages and disadvantages of using different electrical devices for a particular application.

•  to calculate the amount of energy transferred from the mains using,

energy transferred = power × time

(kilowatt-hour, kWh) (kilowatt, kW) (hour, h)

•  to calculate the cost of energy transferred from the mains using,

total cost = number of kilowatt-hours x cost per kilowatt-hour

Skills, knowledge and understanding of how science works set in the context of:

•  Examples of energy transformations that everyday electrical devices are designed to bring about.

•  Examples of everyday electrical devices designed to bring about particular energy transformations.

•  The amount of electrical energy a device transforms depends on how long the appliance is switched on and the rate at which the device transforms energy.

•  The power of an appliance is measured in watts (W) or kilowatts (kW).

•  Energy is normally measured in joules (J).

•  Electricity is transferred from power station to consumers along the National Grid.

•  The uses of step-up and step-down transformers in the National Grid.

•  Increasing voltage (potential difference) reduces current, and hence reduces energy losses in the cables.

1. Why is electrical energy so useful?

2. Give ten devices that we would not be able to use without electricity.

3. What is meant by ‘power’?

4. How many watts in 50kW?

5. Calculate the electrical energy used by:

(a) a 2kW heater in 5 hours

(b) a 400W computer in 4 hours

(c) a 60W light bulb on continuously for 3 days

6. An electricity meter goes from a reading of 4555 to 4985, if units cost 12p each what will be the bill?

7. How many units of electricity are used by:

(a) a 3kW fire in 4 hours?

(b) a 500W electric iron in 30 minutes?

(c) a 10W chargerr in 4 hours?

8. If electrical units each cost 12p, calculate the cost of using each of the devices in Q5 & Q7.

9. What is the ‘National Grid’?

10. Why are transformers needed in the National Grid?

11. What do step-up transformers reduce?

12. What electrical quantity causes energy loss to heat in cables?