## QUESTIONSHEET 1

(a)	3 A / 2 			1 						1

=  1.5 A			1						1

(b)	6 V					1						1

(c)	resistance = V / I	1					1

=  6 / 1.5			1						1

=  4 			1							1

TOTAL  / 6

## QUESTIONSHEET 2

(a)	5   +  10 		1						1

= 15 			1							1

(b)	I = V / R			1						1

= 3 / 15			1						1

= 0.2 A			1						1

Units are essential in calculations.  Sometimes examination questions give a blank space for the answer followed by the unit.  Sometimes no unit is given and you are expected to supply it.  It is a common mistake to leave the units out altogether – don’t get into this bad habit.

TOTAL  / 5

## QUESTIONSHEET 3

6v

(a)

2 

5 

4 

symbols correct	1					1

2  and 4  resistors in parallel	1			1

5  resistor in series	1					1

(b)	resistance of parallel resistors given by

1Rt = 1R1 + 1R2	1				1

= 12 + 14		1						1

Rt = 43Ω		1						1

so total resistance = 43Ω + 5 = 6 13	1			1

(c)	6613					1						1

= 0.95 A			1						1

For parallel resistors, it is sometimes easier to use the expression  Product / Sum for the total resistance.

i.e. here we get 2 × 4 / (2 + 4)  = 8 / 6  =  4/3 

TOTAL  / 9

## QUESTIONSHEET 4

(a)	(i)		Power of lamp is 6 W	1						1

when used on 12 V 	1						1

(ii)		6 W  =  6 J/s	1							1

5 minutes = 300 s	1						1

energy transferred  =  300  6	1					1

= 1800 J	1							1

(b)	(i)		I  =  P / V   =  6 / 12	1						1

=  0.5 A	1							1

(ii)		R  =  V / I   =  12 / 0.5	1						1

=  24 		1							1

(c)	No difference	1							1

24 V shared between 2 lamps / pd across each lamp is 12 V	1

Power is the rate of transferring electrical energy into other forms – in this case, heat and light.  A 6 W lamp transfers 6 Joules of electrical energy into heat & light each second, since 1 W = 1 J/s.

The formula  P = I / V is one you need to know.

TOTAL  / 12

## QUESTIONSHEET 5

(a)	(i)		0.6  10/20	1						1

= 0.3 A		1						1

(ii)		0.6 + 0.3	1						1

=  0.9 A	1						1

(b)			V = I R		1						1

0.6  10 or 0.3  20	1					1

=  6 V		1							1

Current is inversely proportional to the resistance as long as the pd is constant. So half the current flows through the resistor that is twice as big.

TOTAL  / 7

## QUESTIONSHEET 6

(a)	1R = 110 + 110 = 15	1					1

R = 5 				1						1

(b)	10 + 5				1						1

= 15 			1						1

(c)	current through single resistor  = V / R = 6 / 15	1

=  0.4 A			1						1

current through parallel resistors = 0.2 A	1

In many questions, the value of the parallel resistors is the same.  In this case, the total resistance is half the value of one.

TOTAL  / 7

QUESTIONSHEET 7

(a)	(i)		2 + 5 + 10	1							1

= 17 		1								1

(ii)		1/R = 1/2 + 1/5 + 1/10	1						1

= 8 / 10 	1							1

R = 10 / 8 = 1.25 	1						1

(b)	Current larger in (ii)	1						1

current inversely proportional to resistance	1			1

smaller resistance means larger current	1			1

TOTAL  / 8

QUESTIONSHEET 8

(a)	1R1 = 16 + 112 = 14 ⇒ R1= 4	1						1

1R2 = 13 + 16 = 12 ⇒ R2 = 2	1

Total resistance = 2 + 4 = 6	1					1	 						1

(b)	I = V / R			1							1

=  12 / 6			1							1

= 2 A				1								1

(c)	R = 4 + 3			1							1

=  7					1							1

I  =  12 / 7  or 1.7 A	1						1

(d)	R = 3 + 6  =  9 	1						1

I = V / R  =  12 / 9	1						1

=  1.33 A			1							1

TOTAL  / 11

QUESTIONSHEET 9

(a)	C						1						1

(b)	(i)	a complete path	1					1

for electricity to flow	1					1

when connected to a voltage supply	1			1

(ii)	A & D (both)	1						1

No electricity flows in the last circuit, because the cells are the opposite way round from each other,

so they cancel out.

TOTAL  / 5

QUESTIONSHEET 10

(a)	switch shown in either of the positions indicated	1

1.5 A

A1

SWITCH

SWITCH

A2

V

(b)	Parallel			1						1

(c)	1.5 / 3				1							1

=  0.5 A			1								1

(d)	12 V					1								1

TOTAL  / 5

QUESTIONSHEET 11

(a)	Resistance in circuit alters current	1				1

ammeter resistance must be low so it doesn’t affect

current flow		1						1

(b)	(i)		In parallel across component	1					1

(ii)		voltmeter must have high resistance	1				1

little current flows through voltmeter	1				1

most current flows through component	1			1

For two parallel resistors, 1/R  =  1/R1  +  1/R2.  So if one resistor (the voltmeter) R2 has a high resistance, 1/R2 is very small and the total resistance is almost the same as without the voltmeter.

TOTAL  / 6

QUESTIONSHEET 12

x

x

x

x

x

x

x

x

0

0.1

0.2

0.3

0.4

0.5

0.6

0.7

0.8

0.9

1.0

1.1

1

2

3

4

5

6

7

I/A

pd/v

(a)

axes labelled + units	1						1

correct plotting	2						2

smooth curve drawn	1						1

(b)	(i)		no			1							1

(ii)		the graph is not a straight line	1					1

or V / A    R	1							1

(iii)	filament lamp	1							1

TOTAL  / 8

QUESTIONSHEET 13

(a)	Voltage is proportional to current	1

for a metal conductor at constant temperature	1

A

V

Test component

(b)

circuit symbols correct	1

ammeter in series	1

voltmeter in parallel	1

variable resistor in series	1

TOTAL  / 6

QUESTIONSHEET 14

(a)	A  =  metal wire	1						1

B  =  diode		1							1

C  =  lamp filament	1						1

(b)	A						1								1

(c)	C						1

An ohmic conductor obeys Ohm’s law, so a graph of I against V must be a straight line through the origin.

The gradient of graph C decreases so the resistance must be increasing.

TOTAL  / 5

QUESTIONSHEET 15

(a)	(i)

MACHINE

two switches	1							1

in series	1							1

with machine	1							1

(ii)		each switch must be able to turn off machine	1			1

(b)	switches not in series	1						1

each must be able to switch light on or off	1			1

needs two parallel parts of circuit	1				1

TOTAL  / 7

A

QUESTIONSHEET 16

(a)	Set up Ohm’s law circuit 	1				1

measure pd across wire and current through it	1

Test component

measure length of wire	1					1

change length of wire and repeat experiment	1						1						1

V

(b)	(i)

Length (cm)

Resistance ()

(ii)		as length increases	1

resistance increases

directly proportional	1

(iii)	as length varies, resistance varies	1				1

current varies	1					1

and light dims or brightens	1					1

The first two or three marks of this question could be obtained by drawing a labelled diagram.

TOTAL  / 12

QUESTIONSHEET 17

(a)	A  =  battery		1						1

B  =  lamp/light bulb	1					1

C  =  switch		1						1

D  =  variable resistor	1					1

(b)	varies current in a circuit / acts as dimmer switch	1

You must be able to recognise circuit components.  Look at the syllabus or ask your teacher which ones you must know.  Note that a battery is made up of two or more cells joined in series

TOTAL  / 5

QUESTIONSHEET 18

(a)	(i)		1.5 V		1								1

(ii)		0 V			1							1

(iii)	3.0 V		1								1

(iv)	1.5 V		1								1

(v)		1.5 V		1								1

(b)	car bulb needs 12 v	1						1

(c)	increases			1							1

TOTAL  / 7

QUESTIONSHEET 19

(a)	silver too expensive	1						1

(b)	copper is too heavy	1						1

(c)	rubber and PVC	2					1

(d)	water will conduct electricity	1					1

would get shock with wet hands	1				1

TOTAL  / 7

QUESTIONSHEET 20

(a)	One connected after another in line	2			1

(b)	240 / 20			1							1

=  12 V			1								1

(c)	bulb from 40-bulb set uses 6 v	1				1

higher voltage would cause it to blow	1				1

(d)	(i)		fuse would blow repeatedly	1					1

(ii)		fault would not blow fuse	1					1

could be dangerous to operator	1				1

TOTAL  / 9