High Demand Questions	QUESTIONSHEET 1

- Here is a diagram of an electric bell.

A

B

C

D

E

Explain how the bell works by referring to the parts labelled A – E.

(7)

[7]

TOTAL  / 7

High Demand Questions 	QUESTIONSHEET 2

(a)	A large electromagnet is used in a scrap yard to move old cars.

(i)		Name a suitable material for the electromagnet.

(1)

[1]

(ii)		Explain your choice.

(2)

[2]

(b)	A car of mass 1600 kg can be just be lifted.

What is the least force that the electromagnet must use to lift the car?

(1 g = 10 N/kg)

(2)

[2]

TOTAL  / 5

- High Demand Questions 	QUESTIONSHEET 3

Describe a simple electric motor.

Include a simple diagram in your answer.

(7)

[7]

TOTAL  / 7

High Demand Questions 	QUESTIONSHEET 4

(a)	An electric motor is connected to the external circuit via a split ring commutator.

pivot

magnet

coil

#### A

N

S

split-ring

commutor

#### B

carbon brushes

Explain  the purpose of this.

(3)

[3]

(b)	(i)		How could the motor be used as a generator ?

(1)

[1]

(ii)	The current produced would be a varying direct current (dc).

What changes would be needed for the generator to produce an alternating current (ac) instead ?

(2)

[2]

TOTAL  / 6

- High Demand Questions 	QUESTIONSHEET 5

(a)	The diagram shows a relay circuit used in a car starter motor.

starter relay

iron core

heavy current contacts

made of iron

starter motor

car battery (12V)

thin wires

ignition switch

solenoid

thick wires

What happens when the ignition is switched on ?

[3]

(b)Why is the battery always connected to the starter motor with very thick cables ?

(1)

[1]

(c)Draw a circuit diagram showing the battery connections.

[3]

TOTAL  / 7

- High Demand Questions 	QUESTIONSHEET 6

turbine blades

oil

leads to an oscilloscope

solenoid

magnet in the rim

or the wheel

poles of the magnet

An electromagnetic flow meter

(a)	The diagram below shows a device that could be used for measuring the rate at which water flows through a

pipe.

Explain why a reading is obtained on the meter when the water flows in the pipe.

(4)

[4]

(b)	State the effect on the meter of

(i)		increasing the flow rate of the water

(1)

[1]

(ii)		making the water flow in the opposite direction

(1)

[1]

TOTAL  / 6

- High Demand Questions 	QUESTIONSHEET 7

(a)	To avoid cutting through cables, a gas company uses a device to detect their position before digging holes.

X

Y

The magnetic field produced by the cable induces a voltage across the coil. This is registered on a meter attached to the device.

The device cannot detect cables that carry a steady direct current. Explain the reason for this.

(2)

[2]

(b)Why is it better to use the coil vertically than horizontally?

(2)

[2]

(c)State two ways in which you could get a larger reading on the meter.

1. 	[1]								(1)

2. 	[1]							(1)

TOTAL  / 6

- High Demand Questions 	QUESTIONSHEET 8

(a)	The heating effect of an electric current is given by

H = I2R

where I = current, R = resistance.

If the power generated from a power station is 150 kW at a voltage of 275 kV, calculate the current in

the cable.

(4)

[4]

(b)	(i)		The resistance of the cable is 0.01  per km.

Calculate the total resistance of a 200 km length of the cable.

(2)

[2]

(ii)		Find the power loss in the cable over 200 km.

(2)

[2]

TOTAL  / 8

Medium Demand Questions 	QUESTIONSHEET 9

(a)	A wire is positioned between two magnets. The wire is connected to a battery.

motion

N

S

flexible

wire

to low voltage

high current supply

Describe what happens.

(2)

[2]

(b)	(i)		The connections are reversed.

Describe what happens now.

(2)

[2]

(ii)		The battery is replaced by a power supply that gives an alternating current.

What is observed ?

(1)

[1]

TOTAL  / 5

- Medium Demand Questions 	QUESTIONSHEET 10

(a)	Chloe makes an electromagnet by coiling a length of wire around a pencil. What must she do to make it work ?

(1)

[1]

(b)	(i)		Chloe is given some paper clips and asked to test the strength of her electromagnet.

How could she do this ?

[2]

(ii)		State two ways in which Chloe could increase the strength of her electromagnet.

1. 	[1]								(1)

2. 	[1]									(1)

TOTAL  / 5

- Medium Demand Questions 	QUESTIONSHEET 11

(a)	A coil of wire is attached to a battery.  An electric current flows through it.

circular coil

piece of card

The coil acts like a bar magnet.

Draw the magnetic field pattern around it.	[2]						(2)

(b)	(i)		A compass needle held near to the left-hand end points away from the coil.

What does this tell you ?

[1]		[1]

(ii)		Show the direction of the magnetic field lines on your diagram.	[1]			(1)

TOTAL  / 4

- Medium Demand Questions 	QUESTIONSHEET 12

(a)	A simplified electric motor consists of a coil of wire connected to a battery and positioned between two

simple d.c. motor

magnets (see diagram).

coil

b

c

S

N

a

d

brush (fixed)

brush

(fixed)

commutator

(rotates with coil)

(i) 		Use Fleming’s left-hand rule to predict the direction of the force on each arm of the coil.

(2)

[2]

(ii)		State the direction of motion of the coil.

(1)

[1]

(b)	(i)		After half a turn the coil will stop.

What piece if equipment do you need to use to keep the coil moving ?

(1)

[1]

(ii)		What does the piece of apparatus do ?

(2)

[2]

TOTAL  / 6

- Medium Demand Questions 	QUESTIONSHEET 13

(a)	The diagram shows a coil of wire connected to a sensitive ammeter.  A bar magnet is moved towards the coil.

#### N

#### S

sensitive meter

coil

What is observed on the ammeter?

(1)

[1]

(b)	What is observed if:

(i)		the magnet is removed from the coil?

(1)

[1]

(ii)		the magnet and coil move in the same direction at the same speed?

(1)

[1]

(c)	(i)		What name is given to the phenomenon referred to in (b)?

(1)

[1]

(ii)	State three factors which determine the size of the current produced.

1. 	[1]

2. 	[1]

3. 	[1]									(3)

TOTAL  / 7

Medium Demand Questions 	QUESTIONSHEET 14

(a)	Some bicycles are fitted with a dynamo.

What is the purpose of a dynamo ?

1

[1]

(b)	It is not a good idea to have only lights which are powered by a dynamo.

Why not ?

[2]							2

(c)	A bicycle dynamo has a coil that is stationary, whilst the magnet turns inside it when you turn the pedals.

(i)		How does this differ from a simple ac generator ?

1

[1]

(ii)	What advantage does the dynamo arrangement have ?

2

[2]

TOTAL  / 6

- Medium Demand Questions 	QUESTIONSHEET 15

(a)	The National Grid is made up of a network of cables transmitting electricity from power stations

across the country.

A typical power station generates electricity at 25000 V.

The electricity is actually transmitted at 400 000 V.

(i)		How is this achieved ?

(2)

[2]

(ii)		Why is this done ?

(3)

[3]

(b)	(i)		Why might you see a notice warning that it is dangerous to fly kites near to overhead voltage cables ?

(2)

[2]

(ii)		Why is it safe for a bird to perch on an overhead voltage cable ?

(3)

[2]

TOTAL  / 9

- Medium Demand Questions 	QUESTIONSHEET 16

(a)From what material is the core of a transformer made ?

(1)

[1]

(b)	What is the purpose of the core ?

(1)

[1]

(c)	A transformer will only work on ac and not on dc.

Explain why.

(3)

[3]

TOTAL  / 5

Low Demand Questions	 QUESTIONSHEET 17

(a) A wire is connected to a low voltage supply.  A current flows as shown and a magnetic field is produced.

Direction of current

Low

voltage

supply

State two ways in which the magnetic field could be detected.

1.

[2]										(2)

2.

[2]										(2)

(b)Make a sketch of the magnetic field pattern.

Show the direction of the magnetic field.						(2)

[2]

TOTAL  / 6

- Low Demand Questions 	QUESTIONSHEET 18

(a)	What is meant by an alternating current ?

(2)

[2]

(b)Which of the following graphs show an alternating current ?

1

½

_

0

current

+

current

#### A

time

in secconds

#### C

time

current

current

#### B

#### D

time

time

[2]				(one line)								(2)

TOTAL  / 4

- Low Demand Questions 	QUESTIONSHEET 19

Below is a block diagram of a fossil fuel power station.

BOILER

STEAM

TURBINE

GENERATOR

(a)(i)		What is meant by a fossil fuel?

[1]

(ii)	Name two commonly used fossil fuels

2

[2]

(b)	Explain how the power station works

6

[6]

TOTAL  / 9

Low Demand Questions 	QUESTIONSHEET 20

(a)	Name three machines, which make use of motors.

3

[3]

(b)	Electric motors make use of carbon brushes.

What two properties of carbon make it a good material to use?

2

[2]

(c)	State one way in which a motor can be made more powerful.

2

[2]

TOTAL  / 7