E 1.10

INVESTIGATING HOW THE RESISTANCE OF A PIECE OF WIRE

DEPENDS ON ITS CROSS SECTIONAL AREA

- Look at the title of the investigation and make a prediction. How does the Resistance change when the thickness, or cross sectional area, changes? You do not need to back up your prediction with theory.

- Set up the following circuit.

- Take a piece of wire about 125 cm long and cut it into 5 pieces approximately 25 cm each. Use the piece from your other experiment.

- Take one piece of the wire and attach the 2 crocodile clips to it.  Slide the crocodile clips apart along the wire until there is 20 cm of wire in the circuit. This wire will have a cross sectional area of one unit.

- Set the scale on the multi-meter (ohmmeter) to 200 ohms.

- Record the resistance of 1 wire.

TO CHANGE THE CROSS SECTIONAL AREA OF THE WIRE YOU WILL NOW CONNECT THE OTHER WIRES IN PARALLEL WITH THE FIRST. TWO WIRES GIVES AN AREA OF 2 UNITS, THREE WIRES, AREA = 3 UNITS, ETC.

- Take the 2nd wire, place it alongside the first and twist them together. Attach 2 crocodile clips to the twisted wire and slide the crocodile clips apart along the wire until there is 20 cm of twisted wire in the circuit.

- Record the resistance of 2 wires.

- Repeat again for 3, 4 and 5 wires.

- Enter your results in a suitable table

- Plot a suitable graph(s) and draw a conclusion.  Try to explain your results using your knowledge of electricity and atomic structure. Compare your conclusion with your original prediction.

- Write a brief evaluation of both experiments.