1.	(a)	A;	1

(b)	B;	1

(c)	D;	1

(d)	They go out ( or OWTE ) no mark
the circuit is broken/incomplete/(they were) in series;	1

(e)	8 (ohms);	1

[5]

2.	current  =;[In any correct form]

=;

= 0.08 A;	3

[3]

3.	(a)	(i)	240 (volt);	1

(ii)	V = I × R;
6 = 10 × I;
I = 0.6 A;	[Bald, correct answer scores 3 marks]	3

(iii)	0.6 (A);	[Allow ecf from (a)(ii)]	1

(b)	(i)	A description to include:
	1.	electrical;	2
	2.	to light (and heat);
[Allow chemical to electrical to light for 1 mark only]

(ii)	A calculation to include:

either
	energy	= 240 × 0.6 × 12 × 12 or 144 × 3600;;
			[Allow 240 x 0.6 x 12 x 12 for 1 mark]
		= 7.465 × 107 J;

or 
	energy	= 144 × 12 × 12 × 3600;;
		[Allow 144 × 12 × 12 for 1 mark]
		= 7.465 × 107 J;	3

[Accept 20.736 kW h / 20 736 W h for 3 marks]
[Allow 20 736 J for 2 marks]
[20.74 J gains no marks]
[Allow ecf from part (a)(ii)]

(iii)	A calculation to include:
	1.	cost = ;
		[Allow ecf from part (b)(ii)]	2
	2.		= 186p / £1.87;

[Accept kW h argument if candidate shows 1 kW h = 3.6 MJ]

[12]

4.	(a)	3;	1

(b)	to give a greater current/voltage/power/brighter lamp;
[Ignore energy]	1

(c)	symbol correct (circle with a V inside only);
position correct (in parallel with battery);	2

(d)	would get less bright;
[Reject go out]	1

[5]

5.	(a)	(i)	electron;	1

(ii)	arrow B to A;
opposite charges attract / attracted towards positive /
repelled from negative;	2

(iii)	(electric) current;	1

(b)	(i)	A calculation to include:
1.	Q = It; [May use C]
2.	   = 0.5 × 5 × 60;
3.	   = 150;	3
[If 2.5 given as answer then 2 marks max]
[Equation can be implied by the numbers]

(ii)	12;	1

(iii)	A calculation to include:
1.	total energy	= 150 × 12;
		= 1800;
[Allow ecf from part (b)(i)]
[Accept an answer of 30 for 2 marks]

Or using
E	= Vlt method:
E	= 0.5 ×12 × 5 × 60;
	= 1800;
[Allow ecf from 1st line]
[Allow an answer of 30 for 1 mark]	2

[10]

6.	(a)	A circuit diagram to include:
1.	ammeter in series – labelled / symbol
2.	voltmeter in parallel with lamp (and ammeter) – labelled / symbol
3.	power source and complete circuit (ie no obvious gaps)
	(circuit can have a switch / variable resistor / resistor)
deduct a mark for each omission or incorrect placement of component	max 2

(b)	(i)	A description to include two from:
1.	(when first switched on) it is at a low temperature;
2.	it has a high resistance;
3	therefore low current through lamp;	2

(ii)	An answer to include two from:
1	(as temperature increases),resistance of the thermistor lowers;
2	current increases;
3	this happens gradually / slowly;
4	this protects the filament from a current surge;	2

(iii)	1200 (ohms)	1

(iv)	Total resistance = 1200 + 300 = 1500 (ohms);
I =      seen or implied
I =  = 0.16 (A);
[Allow ecf for incorrect value of R]	3

[10]

7.	(a)	(i)

battery    or    or

series circuit; (-1 for obvious gaps, more than 1mm)
only acceptable addition in parallel is a voltmeter. ignore extras
correct symbols;; (-1 for incorrect symbol or omission)
up to max 2. [only penalise lamp once, ignore extras]	3

(ii)	thermal;	1

(b)	(i)	fan rotates / works / blows / (heater off) cold air given out /
/ cold air blown out;

(ii)	no current in the circuit / nothing / no effect;

(iii)	fan rotates / works / blows and the heater is on/ hot air given
out / both work / hot air is blown out;	3

[7]

8.	(a)	(i)	(in light) resistance decreases / low resistance     [ ignore no
resistance]
current increases / current high;
[Ignore switching on the current]		2

(ii)	= 0.015 A / amps / 15 mA
equation or substitution;
correct calculation and unit;		2

(b)	(i)	brightness decreases / dimmer (as the resistance increases)	1

(ii)	Any two from:
(as current increases) the lamp filament gets hotter;
and its resistance increases ;
 is not constant / increases / rate of increase of V greater than
rate of I ;
Ohm’s Law is not applicable / obeyed;
[Ignore references to direct relationships between V and I eg
“as the voltage increases, the current increases”.]	2

[7]

9.	(a)	A suggestion to include:
	electrons;
	pass through tyres to earth;	2
[Allow aircraft is earthed for 1 mark]

(b)	Q = I × t / I =

=

= 4×10–4 A;	4

[5]

10.	(a)	0.057 (A);	1

(b)	R = V ÷ I;

= 2.5 (V) ÷ 0.057 (A);

= 43.9  ;	3

allow full marks for a correct calculation based on an incorrect
reading from the graph.

(c)	the current increases ;
while the voltage stays the same
showing that the resistance decreases	3

(d)	the thermistor is heated by the current passing in it	1

[8]

11.	(a)	(i)	current shows unchanged frequency;
half-wave rectification shown;

(ii)	shows half-waves missing in i);	2

(b)	(i)	1.2 – 1.6 A/amps inclusive;	1

(ii)	R = V/I;	1
    = 0.7 V/1.2 – 1.6 (or any other answer from (i)
    = 0.44 – 0.58 /ohms; (unit essential for last mark)	3

(iii)	initially very high or infinite/remains the same or is constant up
to 0.6 V or whilst there is no current; starts to decrease at 0.6 W
when the current starts to increase;	2

[9]