# SCHOOL OF ENGINEERING AND DESIGN

##### Electronics and Computer Engineering

# January 2006

Module Title – Multimedia Signal Processing

Module Code- EE3052B

# Mid-Session Test January 2006

Time allowed 1 Hour plus 10 minutes reading time

Answer Three out of five questions

School approved calculators are only allowed

Examiners:  Prof. Saeed Vaseghi

Special Stationery Requirements: None

1.

- With the aid of a diagram, or otherwise, express the relationships between the following forms of Fourier analysis: the complex Fourier series, the Fourier transform of continuous-time signals, the Fourier transform of discrete-time signals and the discrete Fourier transform (DFT).

[4 Marks]

- With the aid of a diagram, or otherwise, explain the relationships between the Laplace transform, the z-transform and the discrete Fourier transform (DFT).

[4 Marks]

- Explain how the frequency spectrum X(f) and the bandwidth of a signal are affected by scaling the time axis by a factor of , i.e. from x(t) to x(t), for the following cases: (i)   >1 and (ii)   <1.

[4 Marks]

- An audio signal has a triangular-shaped spectrum with a bandwidth of 4 kHz. Sketch the spectrum of the signal after it is sampled at the sampling rates of (i) 8 kHz and (ii) 4 kHz.

For the case when an audio signal is sampled at 4 kHz, what frequency components of the original signal will be heard at 1 kHz. What is the name of this type of distortion.?

[8 Marks]

## 2.

Assume the signal x(t) is sampled at a rate of Fs=48 kHz and converted to a discrete-time signal x(m).

- Calculate the sampling time interval Ts and the maximum allowable frequency content of the input signal x(t); Fmax.

[4 marks]

The discrete Fourier transform (DFT) equation is given by

X(k)=m=0N−1x(m)e−j2πNkm 	   k = 0, . . ., N1

- Calculate the minimum value of the number of samples N required for a DFT frequency resolution of f=50Hz. What is the corresponding time resolution  for this value of N?

[6 marks]

- For the value of N=4 express the DFT equation in terms of a linear matrix transform with an N-element input vector and N-element output vector.

[4 marks]

- Explain what is meant by the term ‘stationary’ as in the context of a stationary signal process. What is the implication of the stationary assumption in the choice of DFT window length N. What value of N is typically used in the DFT analysis of speech signals.

[6 marks]

## 3.

- Sketch the general form of the shape of the frequncy response of a 2nd order discrete-time filter assuming that its z-transfer function is composed of: (i) A pair of complex zeros and (ii) a pair of complex poles. Explain the effect of an increase or a decrease in the pole radius on the bandwidth of the frequency response of the filter.

[5 Marks]

- Using the inverse Fourier transform method, design two discrete-time finite impulse response quadrature mirror filters to divide the input signal into two equal bandwidth signals. Derive the equation for the impulse response of the filters. Note, first design a digital lowpass filter; and then derive a digital highpass filter from the lowpass filter.

[10 Marks]

- With the aid of a sketch show how the low pass and high pass filters designed in (b) can be repeatedly used in a tree-like structure to divide a signal bandwidth into three subbands with the two lower bands having a bandwidth of half those of the upper band.

[5 Marks]

4.

- Explain why each of the following signals are considered important in the analysis and design of communication signal processing systems:

(1) Sinusoidal signals,

(2) Impulse (also known as Delta or Dirac) function,

(3) Rectangular signal,

(4) White noise.

Give an example of the applications of each type of signal.

[12 Marks]

- Explain what property of a signal is described by its phase.

Describe the constraint on the coefficients of a linear phase finite impulse response filter.

Assuming that a cosine wave x(t)=cos(2ft) is input to a linear filter with a frequency response of , obtain the filter output signal.

[4 Marks]

- Explain how impulse functions and delay elements can be used to model the acoustics reflections from the six walls of a room with a linear filter, assume the time taken for the sound waves to reach the microphone via each reflected path is i  and the path attenuation is Ai.

[4 Marks]

5.

- State what is meant by the term ‘linear time-invariant’ filter and explain the principle of superposition and its relation to convolution.

[4 Marks]

- State the three different methods of representation of a filter function.

[3 Marks]

(c) Figure 1 shows two discrete-time input signals and the impulse response of a linear time-invariant discrete-time filter. Using the principles of linearity and superposition, obtain the outputs of the filter in response to the two discrete-time input signals shown in Figure 1. Hence obtain the output of the filter in response to the sum of the two input signals.

[13 Marks]

Figure 1